Four Door Access Controller

ASC1204C

Features

- Support 100,000 valid cards & 300,000 records

- Support multiple cards

- Support card, password, fingerprint and combination

- TCP/IP or RS-485 interface to PC

- Wiegand or RS-485 interface to readers(2/4 door)

- Door time out alarm, intrusion alarm, duress alarm and tamper alarm

- Anti-pass back, multi-door interlock, multi-card open

- Support 128 efficient time and holiday schedules

- Watch dog function ensure device free from halting

- Surface mounted installation

- Internal battery

Technical Specifications

| Model |  |  | ASC1204C |
| --- | --- | --- | --- |
| System |  |  |  |
| Main Processor |  |  | 32 Bit |
| Memory |  |  | 16M(32M External) |
| Input/Output Ports |  |  |  |
| Input |  |  | Inputs x 9 ( door sensor x 4, exit button x 4, alarm x 1) |
| Output |  |  | Relay x 5(door lock x 4, alarm x 1 ) |
| Reader |  |  | Reader x 4 |
| Capacity |  |  |  |
| Cardholders/Events |  |  | 100,000/150,000 |
| Function |  |  |  |
| User Management |  | General, VIP, Guest, Patrol, Black list, Force |  |
| Alarm Function |  |  | Door time out alarm, Intrusion alarm, Duress alarm, Tamper alarm |
| Schedule Setting |  |  | Support 128 groups of schedule, 128 groups of period and 128 groups of holiday period |
| Interface |  |  |  |
| To PC | RJ45(10/100M)/RS485 |  |  |
| To Reader | Wiegand/ RS485 |  |  |
| Genreal |  |  |  |
| RTC |  |  | Yes |
| Ingress protection |  |  | IP55 |
| Power Supply |  |  | 9-15DC/500mA |
| Internal Battery |  |  | 1204C Support |
| Working Environment |  |  | -30℃-+60℃ |
| Working Humidity |  |  | ≤95% |
| Dimension(W×D×H) |  |  | 280*320*114mm |
| Weight |  |  | 2Kg |

System Connection

Dahua Technology Co., Ltd.

1187 BinAn Road, Binjiang District, Hangzhou, China

Tel: +86-571-87688883

Fax: +86-571-87688815

Email: overseas@dahuatech.com

www.dahuatech.com

*Design and specifications are subject to change without notice.

© 2015 Dahua Technology Co., Ltd.